中文題目：

標楷體14級字型，粗體；文字置中

英文題目：

Time New Roman 14級字型，粗體；文字置中

流體輔助射出成形充填流動可視化分析與探討

版面設定：

邊界＞上＞2.5cm

Flow Visualization of the Fluid Assisted Injection Molding Process

中文作者姓名：

標楷體12級字型；文字置中

英文作者姓名：

Time New Roman 12級字型；文字置中

中文服務單位：

標楷體12級字型；文字置中

吳逸群1、劉士榮2 
Yi-Chuan Wu, Shih-Jung Liu

1科盛科技 Core Tech System Co., Ltd

2長庚大學機械系教授 Department of Mechanical Engineering, Chang Gung University

中文摘要標題：

標楷體12級字型，粗體；

文字置中

英文服務單位：

Time New Roman 12級字型；文字置中

摘  要

近本研究主要係對高分子液體輔助射出成型件（Water Assisted Injection Molding）之充填過程做可視化(Flow visualization)之研究與探討，用以觀察融膠與水在射出成形過程中之交互現象與充填行為。研究將運用先前由本實驗室自行開發之小型研究用液體輔助系統包括水壓幫浦、壓力控制、程序控制及水針等，及開發一新型可視化射出成形模具及不同幾何形狀模穴模仁包括平板、板厚不同、對稱肋件與補強肋件等四種幾何形狀模具。而研究的問題將包括水壓、水溫、模溫、熔膠溫度、延遲時間等之影響。研究亦將探討不同幾何形狀之影響，此外研究也將同時探討不同高分子材料之流變特性與整體加工之關係與探討溫度與壓力的動態變化。

英文摘要標題：

Time New Roman 12級字型；粗體；文字置中

關鍵詞:   射出成型，液體輔助，革新性射出

中文關鍵詞：

標楷體12級字型；標題加粗；文字靠左對齊；4組~6組關鍵字

## Abstract

英文摘要：

Time New Roman 12級字型；文字靠左對齊

版面設定：

邊界＞左

＞2cm

版面設定：

邊界＞右

＞2cm

This report is devoted to studying the filling phenomena of fluid-assisted injection moldings by using a dynamic visualization technique. Experiments were carried out on an 80-ton injection-molding machine equipped with gas and water injection systems. The material used was semi-crystalline polypropylene material. A flow visualization mold was specially designed and made for this study. A high-speed video camera was used to record the mold filling phenomena of rectangular cavities with different channel geometry and layouts: a flat plate, a plate with two zones of different depths, a plate with symmetric ribs, and a plate with fishbone ribs. The interaction between the assisting media and the polymer melt during molding was observed, and insight into the penetration behaviors of the fluid was summarized. In addition, comparisons have been made between the mold-filling process by gas assisted injection molding and water assisted injection molding.

Keyword: injection molding, water assisted,

英文關鍵詞：

Time New Roman 12級字型；標題加粗；文字靠左對齊；4組~6組關鍵字

版面設定：

邊界＞下＞3cm